Algorithm ALG-03: Butterworth filtering and z-score normalization

Purpose: A zero-phase band-pass keeps the heart-sound band; a z-score removes loudness differences between stethoscopes, gains and chest walls.

Input: signal y at 2000 Hz from ALG-02

Output: filtered, normalized float32 signal z, with what was applied

| Parameter | Value | Read from |
| --- | --- | --- |
| target_fs | 2000 | configs/signal.yaml resample.target_fs |
| low_hz | 20 | configs/signal.yaml filter.low_hz |
| high_hz | 400 | configs/signal.yaml filter.high_hz |
| order | 4 | configs/signal.yaml filter.order |
| zero_phase | true | configs/signal.yaml filter.zero_phase |
| effective_order | 16 | effective_order = 2 * int(order) * (2 if zero_phase else 1) |
| padlen | 27 | filters.default_padlen(design_bandpass(...)) |
| min_padlen | 12 | configs/signal.yaml filter.min_padlen |
| norm_method | zscore | configs/signal.yaml normalization.method |
| remove_dc | true | configs/signal.yaml normalization.remove_dc |
| epsilon | 1e-10 | configs/signal.yaml normalization.zero_variance_epsilon |

1  sos <- Butterworth band-pass, prototype order 4, 20-400 Hz at 2000 Hz, as second-order sections (designed once, cached)

2  if the filter is disabled: x_f <- y

3  else:

4      padlen <- 27 samples, sosfiltfilt's own default for this design

5      if |y| <= padlen: padlen <- |y| - 1

6          if padlen < 12: x_f <- y, flag too_short

7      x_f <- sosfiltfilt(sos, y, padlen): forward then backward, zero phase, effective order 16

8  if normalization is disabled or method is none: return x_f

9  if remove_dc (true): x_f <- x_f - mean(x_f)

10  sigma <- std(x_f) in float64

11  if sigma <= 1e-10: return the input unchanged, flag zero_variance

12  z <- (x_f - mean(x_f)) / sigma as float32 (method zscore)

The quoted order is the prototype's: the band-pass transform doubles it and the forward-backward pass doubles it again, to 16. The two passes square the magnitude response, so each cutoff is attenuated twice as many dB as a single pass would attenuate it.

Implemented by:

src/preprocessing/filters.py:133  src.preprocessing.filters:design_bandpass

src/preprocessing/filters.py:167  src.preprocessing.filters:bandpass_filter

src/preprocessing/filters.py:148  src.preprocessing.filters:default_padlen

src/preprocessing/normalize.py:183  src.preprocessing.normalize:normalize

src/preprocessing/normalize.py:127  src.preprocessing.normalize:remove_dc

src/preprocessing/normalize.py:145  src.preprocessing.normalize:zscore_normalize

PV-MEPCG / PulseVision  |  generated from the implementation  |  regenerate: python scripts/39_export_algorithms.py --ids ALG-03