T04. Preprocessing Configuration

Every preprocessing setting applied before feature extraction, with the configuration key it is read from. Values are reproduced exactly as configured and are not rounded: a rounded filter cut-off is a different filter.

| Stage | Setting | Value | Unit | Config key | Notes |
| --- | --- | --- | --- | --- | --- |
| resample | target sampling rate | 2000 | Hz | resample.target_fs | all four datasets are brought to this rate |
| resample | resampling method | soxr_hq | n/a | resample.method | band-limited rational resampler; naive decimation aliases at 44.1 kHz |
| resample | native rates encountered | 2000; 4000; 44100 | Hz | resample.native_fs | D1 2 kHz (no-op), D3/D4 4 kHz, D2 44.1 kHz |
| resample | channel handling | mean | n/a | resample.mono | all 7,536 corpus files are already mono; no-op fast path |
| filter | type | butterworth | n/a | filter.type | designed in second-order sections for numerical stability |
| filter | design order | 4 | n/a | filter.order | prototype order |
| filter | effective order applied | 16 | n/a | filter.order + filter.zero_phase | bandpass transform doubles the prototype to 8 poles; sosfiltfilt doubles it again |
| filter | low cutoff | 20 | Hz | filter.low_hz | -6.02 dB here, not -3.01, because the filter runs forwards and backwards |
| filter | high cutoff | 400 | Hz | filter.high_hz | -6.02 dB here; well below the 1000 Hz Nyquist at the target rate |
| filter | phase | zero | n/a | filter.zero_phase | zero group delay; S1/S2 timings survive filtering |
| filter | filter enabled | True | n/a | filter.enabled | ablation arms PP-C and PP-D set this false |
| normalization | normalization method | zscore | n/a | normalization.method | per record; no statistic is shared between records |
| normalization | DC offset removed | True | n/a | normalization.remove_dc | applied before normalization |
| normalization | zero-variance policy | passthrough_and_flag | n/a | normalization.zero_variance_policy | a constant record passes through unchanged and is flagged |
| normalization | normalization enabled | True | n/a | normalization.enabled | ablation arms PP-B and PP-D set this false |
| framing | frame length | 512 | samples | framing.frame_length | 256 ms at 2 kHz; shared by quality analysis and the spectral features |
| framing | hop length | 256 | samples | framing.hop_length | 128 ms, 50% overlap |
| framing | window | hann | n/a | framing.window | n/a |

Table T04 -- Preprocessing Configuration

Experiment: n/a (not experiment-bound)

Objective: O3 (preprocessing pipeline)

Source: outputs/02_preprocessing/preprocessing_settings.csv

Generated by PV-MEPCG / PulseVision at 2026-08-30T10:48:02.205785+00:00